MUNRO LP SERIES CENTRIFUGAL PUMP SPECIFICATION

- Munro LP Series Centrifugal Pump

  - The LP series self-priming pump shall be of cast iron construction having a cast iron body, cast iron mounting ring and cast-iron diffuser. The impeller shall be of brass construction, completely machined on all outside surfaces and dynamically balanced prior to pump assembly. The pump shall have a stainless steel wear ring and carbon ceramic seal with buna rubber. The pump body shall be mounted to the mounting ring with stainless steel bolts having a dry socket design and the pump shall have a steel base plate.

  - The LP series pump shall include two 3/4” FPT built-in priming and accessory ports for priming or sensor monitoring and a 1/4” FPT sensor port used to accommodate a temperature sensor. One priming port can be used to accommodate a pressure sensor. The pump shall include two stainless steel 1/4” NPT drain plugs – one each in the upper and lower case chambers. The pump shall have a threaded suction size of 2 inches and a threaded discharge size of

1-1/2 inches.

  - The pump motor shall be built in accordance with NEMA, IEEE, ANDI and AFBMA standards. The motor shall have class B or class F insulation with a temperature rise as specified by NEMA standards. Leads shall be terminated in a connection box and shall be clearly identified. The motor shall be designed for across the line starting. Motor shall be ODP, single-phase / three phase, 60 Hz, and 115V, 208V, 230V, 460V. Motors shall be non-overloading throughout the complete pump curve.

  - The pump shall be capable of providing XXpsig @ XX gallons per minute.

  - The pump shall be model (See models below) as manufactured by Munro Pump, Inc. Grand Junction, CO.

NOTE: Items in bold to be removed or amended to fit the specific project.

115/208/230V single phase models:

3/4hp model – LP07B			1hp model – LP100B

1-1/2hp model – LP150B

208/230/V single phase models:

2hp model – LP200B			3hp model – LP300B
5hp model – LP3005B			2hp high head model – LP1502B

208/230/460V three phase models:

3/4hp model – LP07B3			1hp model – LP100B3

1-1/2hp model – LP150B3		2hp model – LP200B3

3hp model – LP300B3			5hp model – LP3005B

2hp high head model – LP1502B3